|  |  |  | # Work order form |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ## Page 1 of 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Customer information |  |  |  |  |  |  |  |  |  |  |  | Product Family: |  |  |  |  |  |  |  |  |  |  | Rev |  | Date |
| Name |  |  |  | National University Hospital (Singapore) Pte Ltd |  |  |  |  |  |  |  | # LABCULTURE® class ii TYPE B2 biosafety cabinet – 230VAC BIBO SERIES G4 (CUSTOMIZED) |  |  |  |  |  |  |  |  |  |  | # 0 |  | # 20-JUL 2026 |
| Country |  |  |  | Singapore |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PO Number |  |  |  | 4520888363 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  | 14-SEP 2026 |
| ESCO PF Number |  |  |  | SO-S-C26009 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  | 24-SEP 2026 |
| factory information |  |  |  |  |  |  |  |  |  |  |  | electrical requirement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Model Code |  |  |  | LB2-4B8-BIBO-C G4 Customize with: - Full SS316L Construction - BIBO HEPA (Exhaust Filter) - Integration of particle counter has a smooth bore per ISO/TR 14644-21:2023, if possible with surface roughness ≤ 0.5 Ra, please amend the bend radius of the connection to NVP counter to be at least 150mm or sloping - CCTV mounting at the side wall - 2x Magnehelic pressure gauge (for Supply & Exhaust Filter) - 2x EO on the Left wall, 1x EO on the Right wall, with max combined output 5 Amps - Full SS316L support stand - Add on 2 EO on support stand with separate cordset from the BSC for particle counter (max combined output of 10A) - Add on shelf on support stand for particle counter - Remove UV Provision - Remove Service Fixture Provision - Cover Balancing on the Back Refer to GA Drawing: BSC-26-089 |  |  |  |  |  |  |  | Power Supply |  |  | 8 (220-240 V, 50/60 Hz) 1st Amended: Update the GA Drawing for the order. PIC: Chika 2nd Amended: Changed the promise week from WK40-5 to WK44-5 (2026). Already approved by Aldrin. PIC: Chika 1st Amended: Update the GA Drawing for the order. PIC: Chika 2nd Amended: Changed the promise week from WK40-5 to WK44-5 (2026). Already approved by Aldrin. PIC: Chika 2nd Amendment 2nd Amendment |  |  |  |  |  |  |  |  |  |  |
| Work Order Number |  |  |  | 2145510 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Serial Number(s) |  |  |  | 224844, 224845, 224846, 224847 |  |  |  |  |  |  |  | Cordset Qty |  |  | 2 (Note: 1 PC for support stand) |  |  |  |  | Plug Code |  |  | D |  |  |
| Shipment details and Promised date |  |  |  |  |  |  |  |  |  |  |  | SOFTWARE REQUIREMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Promised Date & Week (Ex-Bintan) |  |  |  | 44-5 (2026) |  |  |  |  |  |  |  | Airflow Monitoring Enable |  |  |  |  |  |  |  | Measurement Unit Metric |  |  |  |  |  |
| Weekly Vessel Days |  |  |  | N/A |  |  |  |  |  |  |  | MECHANICAL REQUIREMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Destination Port |  |  |  | SINGAPORE |  |  |  |  |  |  |  | COPPER PIPING INSTRUCTIONS N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mode of Shipment ROAD |  |  |  |  |  |  |  |  |  |  |  | S/Fixture Model |  |  |  |  |  |  |  | S/Fixture Model |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Sidewall |  |  |  |  |  |  |  | Sidewall |  |  |  |  |  |
| Combined With WO(s) |  |  |  | N/A |  |  |  |  |  |  |  | Hole Location |  |  |  |  |  |  |  | Hole Location |  |  |  |  |  |
| Fumigation No |  |  |  | Shipment Terms DAP |  |  |  |  |  |  |  | Pipe Exit |  |  |  |  |  |  |  | Pipe Exit |  |  |  |  |  |
| PLATFORM NAMING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LB2-4B8-BIBO-C G4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1st and 2nd digit Product Line |  | 3rd digit class & type |  |  |  |  | 4th digit Width |  | 5th digit TYPE/country |  |  |  | 6th digit electrical code |  |  |  |  | 7th digit |  |  |  | 8th digit |  |  |  |
| lB – labculture TYPE B |  | 2 – class ii type b2 |  |  |  |  | 4 - 4feet |  | b – type b |  |  |  | 8 - 230V; 50/60Hz |  |  |  |  | bibo |  |  |  | g4 |  |  |  |
| MAIN CABINET ORDER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Specifications |  |  |  |  |  | Model |  | Model |  |  |  |  |  | Model |  | Model |  |  |  |  | Model |  |  | Qty |  |
| Cabinet |  |  |  |  |  |  |  | LB2-4B8-BIBO-C G4 2012144 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| Electrical Outlet (Direct mounted) max 2 |  |  |  |  |  |  |  | EO-HD 5170036 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |
| Telescoping Support Stand |  |  |  |  |  |  |  | STA-4A0-C 5131668 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| SPECIAL ACCESSORIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VHP-Out Port Kit with Cover |  |  |  |  |  |  |  | VHP-OUT PORT KIT WITH COVER LB2 G4-4,5,6FT 5171397 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| cUSTOMISATIONS/REMINDERS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NSF CERTIFIED FOR LB2-4B8 G4, LB2-4B9 G4, LA2-4B8-BIBO G4, LB2-4B9-BIBO G4, LB2-4Y7 G4, LB2-4Y7-BIBO G4, LB2-6B8 G4, LB2-6B9 G4, LA2-6B8-BIBO G4, LB2-6B9-BIBO G4, LB2-6Y7 G4, and LB2-6Y7-BIBO G4 UL CERTIFIED FOR ALL LB2 G4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | Description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PIC |  |  |
| QA | Please take pictures after QA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reynold |  |  |
| Cleaning | Remove the UL and NSF sticker. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Heru |  |  |
| Raised by: |  |  |  |  | Chika Claradhea |  |  |  |  | Hardcopy Counter-Checked by: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Saved by: |  |  |  |  | Melani |  |  |  |  | Designer: (Need to submit drawing and BOM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Checked by CSIS Manager: |  |  |  |  | Kevin Esteves |  |  |  |  | Checked By Prod’n.: |  |  |  |  |  |  | Indra Setiawan |  |  |  |  |  |  |  |  |
| Document Number: FM-OSD-255 |  |  |  |  | Revision: E 10-Mar-2025 |  |  |  |  |  | Revised by: Yandra |  |  |  |  |  | Path: \\bintanfs4\Esco_Operating_System\FM-SD-SOP-WI\Order Sales Processing\FM |  |  |  |  |  |  |  |  |